Last Update: 19 August 2019

Owner: Health, Wellbeing and Safety (HR)

| Portable Electrical Equipment | Yes | No |
| --- | --- | --- |
| 1. Is there damage (apart from light scuffing) to the cable sheath? |  |  |
| 2. Is the plug damaged (eg. the casing cracked or pins bent)? |  |  |
| 3. Are there inadequate joints, including taped joints, in the cable? |  |  |
| 4. Is the coloured insulation of the internal cable cores showing where they enter the plug? |  |  |
| 5. Does the appliance appear to have been subjected to conditions for which it is not suitable (eg. is it wet or excessively contaminated)? |  |  |
| 6. Is there damage to the external casing of the equipment or are there loose screws or parts etc? |  |  |
| 7. Is there evidence of overheating (eg. burn marks or discoloration)? |  |  |
| 8. Is the main on/off switch damaged, does it operate incorrectly? |  |  |
| Notes: |  |  |

An answer “Yes” to any of the above indicates that the appliance is potentially dangerous and must be taken out of use.

| General Electrical Inspection: Visual inspection of electrical equipment may include the following: | Yes | No |
| --- | --- | --- |
| - Flexible cords in good condition ie look for cuts, abrasion or damage |  |  |
| - External components or casing are not damaged (these may form part of the insulation) |  |  |
| - Power and extension cords anchored and separated from other hazards such as liquids, mechanical action or traffic areas. |  |  |
| - Covers, guards, controls, alarms or mechanical safety features are in good condition |  |  |
| - Power outlets and power boards are not overloaded. |  |  |
| - Cords are kept away from traffic areas. |  |  |
| - Electric cords and wiring are not kept under carpets where they are subject to wear and tear |  |  |
| - Electric cords and wiring are not run through windows or doorways where they are subject to abrasion |  |  |
| - Double adapters are not used (power-boards should be used) |  |  |
| - Bar radiators and para-flood lamps are not used for heating purposes |  |  |
| - Appropriate electrical equipment are used in wet areas such as around sinks and in toilets |  |  |
| - Only designated electrical appliances, equipment, or extension cords are used in areas which contain dangerous or hazardous materials (flammable, explosive, or corrosive) |  |  |
| Notes: |  |  |