| Instrument | Zeiss Axioskop2 |  |  |  | Room#nb1.108 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Application: Transmitted light and fluorescence imaging of fixed tissues or cells on slides |  |  |  |  |  |
| User information | All users must be trained prior using this instrument. Please see webpage or contact SoM-BIF facility for further information about training and costs involved. |  |  |  |  |  |
| Webpage: | https://www.micro-shop.zeiss.com/?s=95791363abbd0b&l=en&p=us&f=a&i=10036 |  |  |  |  |  |
| Light source | HAL100 (halogen lamp) |  |  |  |  |  |
|  | HBO100 (mercury lamp) |  |  |  |  |  |
| Filters | DAPI, FITC, Cy3 |  |  |  |  |  |
| Objectives | Type | magnification | NA | Type | Coverslip thickness | Working distance (mm) |
|  | Fluar | 5x | 0.25 | air | 0.17 | 12.5 |
|  | Plan Neofluar | 10x | 0.3 | air | 0.17 | 5.2 |
|  | Plan Neofluar | 20x | 0.5 | air | 0.17 | 2 |
|  | Plan Neofluar | 40x | 0.75 | air | 0.17 | 0.71 |
|  | Acroplan | 100x | 1.3 | oil | 0.17 | 0.29 |
| Camera | Axiocam 506color |  |  |  |  |  |
| software | ZEN |  |  |  |  |  |