| Instrument | Olympus IX71 |  |  |  | Room#ka4.316.1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Application: Transmitted light and fluorescence imaging of fixed tissues or cells on slides |  |  |  |  |  |
| User information | All users must be trained prior using this instrument. Please see webpage or contact SoM-BIF facility for further information about training and costs involved. |  |  |  |  |  |
| Webpage: | http://www.olympusmicro.com/brochures/pdfs/ix71.pdf |  |  |  |  |  |
| Light source | Transmitted: TH-200 |  |  |  |  |  |
|  | Fluorescence: I-RFL-T (mercury) |  |  |  |  |  |
|  | Fluorescence: U-RX-T (Xenon) |  |  |  |  |  |
| Filters | DAPI, FITC, Cy3, TxRED, Cy5, Cy7 |  |  |  |  |  |
| Objectives | Type | magnification | NA | Type | Coverslip thickness | Working distance (mm) |
|  | UPlanFLN | 10x | 0.3 | air | 0.17 | 10 |
|  | UPlanSApo | 20x | 0.75 | air | 0.17 | 0.6 |
|  | LUCPlan | 40x | 0.6 | air | 0.17 | 2.7-4 |
|  | UPlanSApo | 40x | 0.95 | air | 0.17 | 10 |
|  | UplanSApo | 100x | 1.4 | oil | 0.17 | 0.13 |
|  | UplanFLN | 100x | 1.3 | oil | 0.17 | 0.2 |
| Camera | OlympusXM10 |  |  |  |  |  |
|  | DP25 |  |  |  |  |  |
| software | Cell Sense |  |  |  |  |  |